Editorial notes and theoretical material1. calibration tables and examples

Table 1. Residential daily preference profile

| Time period | ωₜ range | Household activity |
| --- | --- | --- |
| 00:00–06:00 | 0.50–0.60 | Night-time base load. Electricity is mainly used for essential appliances such as refrigerators, standby devices, and minimum household services. |
| 07:00–10:00 | 0.90–1.10 | Morning peak period. Increased demand due to breakfast preparation, cooking, water heating, lighting, and household preparation activities. |
| 11:00–16:00 | 0.75–0.85 | Midday period. Household occupancy is typically lower as residents may be away from home. Electricity demand is primarily associated with refrigeration and seasonal heating or cooling loads. |
| 17:00–22:00 | 1.20–1.50 | Evening peak period. Most household members are present, resulting in increased demand for cooking, entertainment, lighting, domestic appliances, heating/cooling, and comfort-related services. |
| 23:00–24:00 | 0.60–0.70 | Transition to night-time operation with gradually declining household activity and electricity demand. |

Table 2. Discomfort-parameter ranges

| Consumer profile | ζ (€/kWh²) | Interpretation |
| --- | --- | --- |
| Highly Flexible Prosumer | 0.02 – 0.05 | Strong willingness to shift demand and participate in demand response programs |
| Typical Residential Prosumer | 0.05 – 0.20 | Moderate flexibility with balanced consideration of comfort and economic incentives |
| Relatively Inelastic Prosumer | 0.20 – 0.50 | Limited willingness to modify consumption patterns without substantial compensation |

2. Retained derivation of the deficit P2P price

The attachment assumes an inverse relationship between price and the supply-to-demand ratio. For a local deficit, it imposes the spot price at SDRt = 0 and the charging/injection reference price at SDRt = 1. The derivation below reproduces that algebra. It requires the relevant reference prices and denominators to be non-zero.

PriceP2P,t=1a1·SDRt+b1

At zero local supply, the first boundary condition gives:

SDRt=0,  PriceP2P,t=Pricespot,t,  b1=1Pricespot,t

At unit supply-to-demand ratio, the second boundary condition gives:

SDRt=1,  PriceP2P,t=Priceprosumer,ch-inj,t,  a1+b1=1Priceprosumer,ch-inj,t

a1=Pricespot,t-Priceprosumer,ch-inj,tPricespot,t·Priceprosumer,ch-inj,t

Substitution recovers Eq. (13):

PriceP2P,t=Pricespot,t·Priceprosumer,ch-inj,tPricespot,t-Priceprosumer,ch-inj,t·SDRt+Priceprosumer,ch-inj,t

Other retained analytical statements

For the unconstrained interior consumption model, the source uses the marginal-utility condition and obtains the demand-price derivative:

∂U∂Et=Priceelec,t,  ωt-a·Et=Priceelec,t

dEgrid,tdPricespot,t=-1a

The source also states the following directional relationship between community battery capacity and the variance of SDR. It is retained as a source proposition, not established here by a new proof or simulation:

∂VarSDRt∂CbatCommunity<0

The capacity-aware charging and discharging quantities, the unconstrained elasticity expression and the source degradation expression remain visible in Eqs. (6), (8), (4) and (16) of the main insert. Repeated inline variable labels and repeated explanatory sentences are not duplicated here.